HORTICULTURAL ACTIVITIES

CONTRACT PERIOD:

01 July 2009 to 30 June 2010 with an option, by the Municipality, to extend, annually, for a further period of two years not exceeding 30 June 2012.

A.	GENERAL SCOPE OF THE WORK

The contract is for the following horticultural work to be undertaken at sites within the boundary of the Nelson Mandela Bay Municipality:

Item 1:	Preparation of tree holes to various sizes

Item 2:	Supply and spreading of topsoil to various depths

Item 3:	Supply and planting of kikuyu and kweek sods only

Item 4:	Supply, planting and maintenance of kikuyu and kweek sods

Item 5:           Supply of Modek shade house sheeting

Item 6            Supply of Agricultural Shade Netting .

Item 7            Supply of tree stakes.

Item 8            Supply plastic plant bags.

Item 9            Suply pots, trays, containers etc.

Item 10          Supply of  flowering annual seed.

Item 11

Annexure A, B and C must be completed and signed by all tenderers.

B.	GENERAL CONDITIONS OF THE CONTRACT

The following general conditions are applicable and should be read together with the specific requirements for the various tender items.

1.	MAKING GOOD

1.1	The contractor shall leave the site/s in a tidy condition, and landscaped to a level after removal of all earth, rubble, etc.

1.2	The area and quality of material excavated on site shall dictate what quantity of topsoil will be required to finalise the repairs to the landscaping where applicable.

1.3	Where cut or fill occurs adjacent to a structure or any playing surface the gradient left by the contractor must not exceed 1:6

1.4	Any topsoil provided by the contractor shall first be approved by the Environmental Services Directorate.

  - Surfaces where construction has taken place must be landscaped with the approved topsoil and grass reinstated.

  - Disturbances of virgin soil requires a permit from the Provincial Department of Economic Affairs, Environment & Tourism in terms of the EIA regulations.  All tenderers shall indicate their source of topsoil and shall submit documentary proof of their compliance with EIA regulations with the tender documents. Failure to supply this proof will automatically disqualify the tenderer.

2.	SAFETY AND PUBLIC LIABILITY

2.1	It is necessary that the contractor shall, at his own expense, ensure that the prescribed safety precautions are taken during the full duration of the construction period and indemnity and safeguard the ……………. Municipality and its employees against all actions, suits, proceedings, claims, demands, costs and expenses whatsoever which may be instituted, brought or sent, or may be incurred or may be payable by the Municipality arising out of or in connection with any damage, death or injury caused or alleged to have been caused by or as a result of any act, omission by the contractor and/or the contractor's employee or employees arising out of work done in connection with or arising out of this contract.

3.	SERVICES

3.1	Although every effort will be made to provide the contractor with the correct information regarding services and other relevant information, it is the responsibility of the contractor to determine the exact location of all underground or other services.  The Municipality will not be held responsible for any claims whatsoever arising from damage to the Municipality's assets or that of any other party caused by the contractor, his staff or equipment, whilst executing this contract.

3.2	Any damage caused by the contractor as per par. 2 and 3, will be for his own account and must be reported immediately to the responsible department.

4.      PAYMENT

4.1.   Authorisation for payment will only be made as per contract and after the area has been inspected by both parties and the officials of the relevant Directorate are satisfied that all aspects pertaining to the specification have been met.

5.	SPECIAL CONDITIONS

5.1	The Municipality reserves the right to inspect the progress and quality of the work at any time during the contract period and to stop a contractor from continuing with the contract if, at any stage, it should be found that the specifications laid down are not being adhered to or if, in the opinion of the responsible Business Unit, work is not being undertaken to the standard required.

5.2.

5.3.   The demand for these services vairy from time to time and will seldom be constant. In this regard the contractor must ensure that he has enough staff and vehicles to carry out the work at all times. The recruitment of extra staff and vehicles will be for the contractors account.

C.	QUALITY OF GRASS SODS AND SPRIGS

The following species are prescribed.

Cynodon dactylon - fine kweek

Pennisetum clandestinum - kikuyu

Local strains of fine kweek are acceptable if vigorous and true to type.  Any strain having adaptations to growing in saline areas such as shortened internode, hairy leaves or blue green colour are unacceptable.

Prior to the acceptance of the tender the Municipality reserves the right to inspect the tenderers source of grass to verify the acceptance thereof. It is anticipated that Council will use about 20Ha of grass per annum. However this figure may vairy drastically due to unpredicted projects and budget.

The Municipality also reserves the right to inspect the approved tenderers source of grass during the course of the tender period and to reject grass that does not comply with the specifications or are not in a good and acceptable condition.

Grass sods must be fresh and vigorous, must consist of equal quantities of rhizomes and stolons.No broken  damaged pieces will be accepted. Grass must contain a reasonable amount of soil bound to the roots (ie the soil must not be shaken off) to ensure that the product does not deteriate after planting.

Grass grown for sodding purposes should be fertilized and renewed on a regular basis and kept pest free. Sods delivered should be true to species and pest free.    Work must commence no later than 10 working days after receipt of the official order number. The contractor will be expected to lay a minimum of ………m² per day. FAILURE TO ADHERE TO THIS A PENALTY OF 5% OF THE TOTAL CONTRACT COST WILL DE DEDUCTED PER DEFAULT DAY.

D.	SPECIFIC REQUIREMENTS OF TENDER ITEMS

ITEM 1 - i	PREPARATION OF TREE HOLES

The contractor will not be required to prepare tree holes in hard rock.

All holes must be square and the required size is:

1m x 1m x 1m (1 m³)

The location and number of holes will be pointed out to the contractor on site.  An official from the responsible Directorate will confirm the size and quality of holes before the contractor fills them with topsoil to the level of the surrounding soil surface. It will be the contractors responsibility to contact the department. The topsoil which is to be supplied by the contractor shall be of fertile quality approved by the responsible Business Unit or his representative.  It shall be free from large roots, stones, refuse, stiff or heavy clays. The contractor shall submit a sample of the topsoil for approval before it is used on site.

ii Council will supply the trees, tree stakes and materials for tying. Tree sizes (max of 100l containers) ,stakes and method of tying may vairy from area to area time to time. Contractor to fill the hole to 100mm below the surrounding surface level. Tree container to be removed without damaging or disturbing root ball and the soil level of the tree to be level with the soil level of the hole. Tree to be in the center of the hole and vertical. Contractor to water immediately after planting with 10 liters water. Sinkholes to be filled by the contractor. Contractor to supply tools.

Work must commence no later than 5 working days after receipt of the official order number. The contractor will be expected to lay a minimum of ………m² per day. FAILURE TO ADHERE TO THIS A PENALTY OF 5% OF THE TOTAL CONTRACT COST WILL DE DEDUCTED PER DEFAULT DAY.

The contractor must complete a minimum of 10 holes / day.

If the tree hole size is altered the contractor will be paid proportionaltely i.e .if the cost of one m³ hole is R100 and the requirement requires a hole of ,5m³ the price will be R50 per hole. Likewise if the requirement is 2m³ hole the price will become R200 per hole.

ITEM 2 -	SUPPLY AND SPREADING OF TOPSOIL

Topsoil shall consist of fertile loamy soil, with a good coverage of natural vegetation obtainable from sources, approved by the responsible Directorate. It shall be free from deleterious matter such as large roots, stones, refuse, stiff or heavy clays and the seeds of noxious weeds which will adversely affect its suitability for grass planting.

The contractor shall submit a sample of the topsoil for approval before it is used on site.  The area will be handed to the contractor after it has been graded to an even and clean but rough condition.

The contractor may use any approved method to produce an even spread of the topsoil.  The contractor must apply the topsoil to the required depth and ensure that the final finish is level.  The contractor must rake and remove all foreign material e.g. roots and stones in excess of 10mm in diameter.

The contractor will be required to spread topsoil on unplanted sites or on established lawns.

The unit of measurement shall be per m² for the various depths of top soiling. The required depths will be 25mm, 50mm, 75mm, 100mm and 150mm.    Work must commence no later than 10 working days after receipt of the official order number. The contractor will be expected to lay a minimum of ………m² per day. FAILURE TO ADHERE TO THIS A PENALTY OF 5% OF THE TOTAL CONTRACT COST WILL DE DEDUCTED PER DEFAULT DAY.

ITEM 3 -	SUPPLY AND PLANTING OF KIKUYU AND KWEEK SODS ONLY

Grass sods shall be fine kweek (Cynodon Dactylon) or kikuyu (Pennistum Clandestinum) as specified.  It shall be nursery grown, cut, delivered and laid within 36 hours.It shall be free of weeds and disease and contain a minimum of 30mm soil.The area to be harvested must be well watered prior to harvesting and handling must be done with care to retain the maximum amount of moisture in the roots.

The contractor shall plant sods on sites where the ground was prepared by the Municipality.  The area where the sods must be laid shall first be well watered by the contractor.

The contractor must apply 40gm/m ²  2:3:2 fertiliser prior to laying sods.

Sods shall be fitted closely together and any hollows must be filled with topsoil to produce an even and smooth surface

After laying, the sods must be well watered continuously during the sod-laying process and rolled to obtain a level surface.

After the completion of the sod laying 40gm/m² L.A.N. fertiliser must be applied. A once-off heavy watering must then be applied immediately after the spreading of the fertiliser.

If the delivery and laying of grass is to be outsourced an official letter must accompany the tender documents stating the company and the capacity of the outsourcee.The responsible council official will only deal with the supplier. Failing to provide this proof will disqualify the tenderer

Work must commence no later than 10 working days after receipt of the official order number. The contractor will be expected to lay a minimum of ………m² per day. Failure to adhere to this a penalty of 5% of the contract cost will de deducted per default day.

ITEM 4 -	SUPPLY, PLANTING AND MAINTENANCE OF KIKUYU AND KWEEK 				SODS

All the requirements/specifications of Item 3 above is also applicable to this item. The minimum period for the maintenance of the grass to establish is 6 weeks after the final laying and the contractor shall allow for this maintenance period.

The whole area must be watered regularly and kept weed free by an approved method to produce a complete grass cover in the shortest possible time.  Standpipes, if required, must be obtained by the contractor from the Waterworks Division at his own expense.  It is likely that a water point could be as far as 100m from the site.  The contractor must make allowances for this.

Where grass fails to grow such areas will be replanted by the contractor at his own expense.  A minimum of two cuts will be undertaken by the contractor before handing over.

No area will be accepted unless a minimum of 90% general coverage has been obtained.

After the six(6) weeks period the site can be handed over. No area will be regarded as handed over unless a certificate to this effect has been issued by the Manager: Environmental Services Business Unit or his representative.

Payment shall be 70% on completion of planting, 30% when the grass is established and a final cut has taken place.

Work must commence no later than 10 working days after receipt of the official order number. The contractor will be expected to lay a minimum of ………m² per day. FAILURE TO ADHERE TO THIS A PENALTY OF 5% OF THE TOTAL

Item5:           Supply of polycarbonate roof sheeting .A layer of UV protection PC must be co – extruded on the weather side. Opal colour in various lengths of 1mm Nominal Thickness. Supplier must have a local backup office with technical expertise. Delivery to …………………………. to be included. The sheets will be used for nursery structures. Sheet lengths will vairy.

Item 6            Supply of Agricultural Shade Netting.

The product must have a SANS 1703:2003 approval. Governament Notice No 1373 of 8th November 2002.

Dimentions - length to be 50m long by 3m wide knitted, woven and plain coloured coated shade netting in the following percentage

shade.30%,40%,50% ,70% ,80% & Knippol. The finish to be uniform make, width, colour & finish. Supplier must have a local back up office

with technical expertise. Delivery to ……………………………… to be included.

Item 7            Supply of tree stakes.

The tree stakes must be tanalith treated.2,4m long with a diameter range of between 32mm – 50mm. Cost to include delivery to the following depots…………………………………………………………………………………….

Item 8            Supply plastic plant bags. Must be virgin plastic, black.

Item 9            Supply pots, trays, containers etc.

Item 10          Supply of  flowering annual seed.

CONTRACT COST WILL DE DEDUCTED PER DEFAULT DAY.

6. 	EVALUATION CRITERIA

90/10

- Cost of works / goods				90

- Equity owned by HDI’s				5

- Equity owned by Women				2

- Equity for disabled persons			1

- Equity for Youth (18-36 yrs)			1

- Equity for NMMM based contractors		1

# ANNEXURE A

# DETAIL OF RESOURCES - FORM TO BE COMPLETED BY ALL TENDERERS

| NO | REQUIREMENTS | NAME OF SUPPLIER | CONTACT PERSON | TEL NO OF CONTACT PERSON | NEGOTIATED PRICE TO CONTRACTOR PER UNIT |
| --- | --- | --- | --- | --- | --- |
| 1 | KIKUYU SODS |  |  |  | R /M² |
| 2 | KWEEK SODS |  |  |  | R /M² |
| 3 | TOPSOIL |  |  |  | R /M³ |
| 4 | TRUCKS |  |  |  | R /DAY |
| 5 | LOADER |  |  |  | R /DAY |
| 6 | SOD CUTTER |  |  |  | R /DAY |
| 7 | GRADER |  |  |  | R /DAY |
| Documentary proof that source of topsoil complies with EIA regulations to be attached |  |  |  |  |  |

ANNEXURE “A”

HORTICULTURAL ACTIVITIES

| ITEM NO. | DESCRIPTION | UNIT | TOTAL PRICE EXCL. V.A.T. | Area A | Area B | Area C |
| --- | --- | --- | --- | --- | --- | --- |
| 1 (a) | Prepare treeholes:1m x 1m x 1m (1m³) No filling with soil | 1 |  |  |  |  |
| (b) | Prepare treeholes:1m x 1m x 1m (1m³) Fill with one cubic meter(m³) of approved topsoil | 1 |  |  |  |  |
| (c) | Prepare treeholes,soil,plant & stake | 1 |  |  |  |  |
| 2 | Supply and spreading of topsoil to various depths |  |  |  |  |  |
| (a) | 25mm thick per m² | m² |  |  |  |  |
| (b) | 50mm thick per m² | m² |  |  |  |  |
| (c) | 75mm thick per m² | m² |  |  |  |  |
| (d) | 100mm thick per m² | m² |  |  |  |  |
| (e) | 150mm thick per m² | m² |  |  |  |  |
| 3 | ## Supply and planting of sods |  |  |  |  |  |
| (a) | Kikuyu sods(no maintenance) | m² |  |  |  |  |
| (b) | Kweek sods(no maintenance) | m² |  |  |  |  |
| 4 | Supply, planting and maintenance of sods |  |  |  |  |  |
| (a) | Kikuyu sods | m² |  |  |  |  |
| (b) | Kweek sods | m² |  |  |  |  |
| 5. | Supply polycarbonate roof sheeting | Running m |  |  |  |  |
| 6: | Supply of Agricultural Shade Net |  |  |  |  |  |
|  | 30% Black | roll |  |  |  |  |
|  | 30% White | roll |  |  |  |  |
|  | 40% Black | roll |  |  |  |  |
|  | 40% White | roll |  |  |  |  |
|  | 40% Green | roll |  |  |  |  |
|  | 50% White | roll |  |  |  |  |
|  | 50% Green | roll |  |  |  |  |
|  | 70% Black | roll |  |  |  |  |
|  | 80% Black | roll |  |  |  |  |
|  | 80% Green | roll |  |  |  |  |
|  | Knippol | roll |  |  |  |  |
| 6 | Supply of polycarbonate roofsheeting | Per running meter |  |  |  |  |
| 7 | Supply of tree stakes. | Per 50 |  |  |  |  |
| 8 | Supply plastic plant bags. | 75mm+ 50 x 180 (width, fold, length) - 50 micron thick 0.8 litres |  |  |  |  |
|  |  | 100 + 75 x 200 - 62 – 1.5 |  |  |  |  |
|  |  | 125 + 100 x 225 – 80 – 2. |  |  |  |  |
|  |  | 150 + 125 x 300 – 100 – 4.5 |  |  |  |  |
|  |  | 175 + 150 x 350 – 100 - 8 |  |  |  |  |
|  |  | 225 + 200 x 475 – 150 - 20 |  |  |  |  |
|  |  | 340 + 210 + 550 – 200 - 40 |  |  |  |  |
|  |  | 350 + 300 x 625 – 200 - 60 |  |  |  |  |
|  |  | 520 + 400 x 860 – 200 - 100 |  |  |  |  |
| 9 | Supply pots, trays, containers etc. | 7.5 cm 0.190 volume |  |  |  |  |
|  | Pot coloured | 9.0 – 0.305 |  |  |  |  |
|  |  | 10.0 – 0.500 |  |  |  |  |
|  |  | 12.5 – 0.795 |  |  |  |  |
|  |  | 15.0 – 1.500 |  |  |  |  |
|  |  | 18.0 – 2.400 |  |  |  |  |
|  |  | 20.0 – 3.700 |  |  |  |  |
|  |  | 25.0 – 7.100 |  |  |  |  |
|  |  | 30.0 – 14.700 |  |  |  |  |
|  |  | 35.0 – 24.200 |  |  |  |  |
|  |  | 43.0 – 43.500 |  |  |  |  |
|  |  | 58.0 – 91.000 |  |  |  |  |
|  | Pot black | 25.0 – 7.100 |  |  |  |  |
|  |  | 30.0 – 14.700 |  |  |  |  |
|  |  | 35.0 – 24.200 |  |  |  |  |
|  |  | 43.0 – 43.500 |  |  |  |  |
|  |  | 58.0 – 91.000 |  |  |  |  |
|  | Shrub pot black | 12.5 – 1.000 |  |  |  |  |
|  |  | 15.0 – 1.600 |  |  |  |  |
|  |  | 16.0 – 2.500 |  |  |  |  |
|  |  | 17.5 – 3.300 |  |  |  |  |
|  |  | 20.0 – 4.900 |  |  |  |  |
|  |  | 23.00 – 5.800 |  |  |  |  |
|  |  | 25.0 – 7.100 |  |  |  |  |
|  | Plant Marker - white | 10 x 2.5 |  |  |  |  |
|  | Polystirene 200cell tray. | 24 gram density,6cm x34cmx67cm.square holes |  |  |  |  |
| 10. | Supply of flowering annual seed. | Must be registered with SANSOR & member of seedling grower association,SANA member,listed with Dept of Agriculture,member of Professional Bedding Plant Grower Association. |  |  |  |  |

PRICES ARE VALID FOR 90 DAYS FROM CLOSING DATE OF TENDER

TENDERER TO STATE %:  _______________ DISCOUNT FOR PAYMENT WITHIN 30 DAYS OF STATEMENT/INVOICE

(THE DATE WILL BE CALCULATED FROM THE DATE OF RECEIPT OF GOODS/RENDERING OF SERVICE)

DELIVERY PERIOD, IN WEEKS, FROM DATE OF RECEIPT OF ORDER: _____________

NAME OF TENDERER:  ______________________________________________________

ADDRESS:  ________________________________________________________________

TEL. NO.: __________________________ FAX NO.:  _______________________________

SIGNATURE OF TENDERER:  ______________________  DATE: ____________________